Modular Offshore Turntable

# Technical Specification

| Turntable construction | ln accordance with existing HICE-design (DNV-GL design-approval) |
| --- | --- |
|  | Foundation c/w 2 tracks of loadbearing wheels (h=508 mm) Track 1 with diameter 6,640 mm and surface 26 m2 Track 2 with diameter 11,520 mm and surface 40 m2 Track 1 contains 18 pcs loadbearing wheels in hinged carrier Track 2 contains 36 pcs loadbearing wheels in hinged carrier Foundation c/w 2 tracks of loadbearing wheels (h=508 mm) |
|  | Central bearing connected by star-frame to track 1/2 Turntable deck consisting of 18 segments, 6mm steel plated Outer/inner walls consisting of 4 m high interconnected SHS120, connected by rolled Bulb-Profile |
| Dimensions | Outside diameter 14.2 m Inside diameter 4.0 m Basket internal height 4.0 m Basket bottom height 0.85 m |
| Drives | Electro-Hydraulic power unit, 2 x 75 kW, 400 V 50Hz 4 x hydraulic HICE-sprocket drive (50% redundancy) approx. 20 kW with fixed ring gear on turntable deck HPU can be operated separately |
| Coating | 1 x primer ‘jotun’ 0.150 mm 1 x topcoat ‘jotun’ 0.050 mm RAL 5010 blue |

# General Arrangement

| General information |  |
| --- | --- |
| HICE lifting system | hydraulic compensation system with 40 mm strake lifting the Turntable deck from 0 mm (fixed, non-rotating) to + 40 mm (maximum strake, rotating) by means of 54 cylinders (stainless rod) divided over 6- sections |
| Weight/Capacity | Mass (empty) approx. 100 t (depends on final lay out) Carrying capacity 1,000 t |
| Load bearing wheels | 54 pcs cast iron wheel 400 mm diameter bearing capacity 20.5 t/wheel |
| Load distribution | the static Uniform Distributed Load (UDL) generated through the 2 foundation track with the turntable loaded to maximum capacity is approx. 12 t/m2 The foundation tracks are 1.2 m wide with a combined surface of approx. 71 m2 |
| General: | DDNV-GL certified |

# Performance Data

| Modular Offshore Turntable |  |
| --- | --- |
| Rotational speed | 1,200 m/hour at centre core (ID) |
| Electrical requirements | 400V/50Hz power supply included |
| Parking brake | 20 seconds from 0 - max. |
| Accelerations with full load | 20 seconds from max. to 0 |
| Braking with full load | 10 seconds from max. to 0 |
| Fast Stop Braking with full load | <5 seconds from max. to 0 |
| Emergency Stop Braking with full load | 1,200 m/hour at centre core (ID) |

# Turntable Basement